INVENTION DISCLOSURE FORM FOR PATENTS

Applicant Name-Marwadi University

- Particulars of Inventors

| Mr./Ms/Dr. | Name (Full) | Department | Designation | Mobile No. | Email | Postal Address |
| --- | --- | --- | --- | --- | --- | --- |
| Mr. | Jash Paritosh Jani | ICT-Diploma | Student | 9773214031 | jash.jani125637@marwadiuniversity.ac.in | Shri Sadguru Watika Airport Road, Maruti Nagar 2, D502 |
| Mr. | Aatman Shah | ICT-Diploma | Student | 9510454363 | aatman.shah128769@marwadiuniversity.ac.in |  |
| Mr. | Vishal Solanki Sir | ICT-Diploma | Professor | 9033746407 | vishal.solanki@marwadieducation.edu.in |  |

- Summery:-

• This project outlines the creation of a custom 3-axis CNC/PCB plotter, with mechanical parts fabricated using an Ender 3 3D printer.

• User designs from Inkscape are translated into G-code (via JS Cut) to control the plotter's movements. • An Arduino Uno microcontroller precisely manages the X, Y, and Z axes for accurate plotting.

• The resulting plotter achieves pen-based drawing or PCB tracing, offering a tailored alternative to commercial options.

• The project demonstrates a cost-effective method for building specialized hardware by integrating 3D-printed components with accessible software and electronics.

- Detail description of the invention:( Answer to all below are required in detail)

  - Problem the invention is solving:

Existing commercial CNC/PCB plotters can be expensive, limiting accessibility for hobbyists, small businesses, and educational institutions. Traditional manufacturing methods for these devices often involve complex machining and assembly processes, contributing to the high cost. There is a need for a more affordable and accessible solution for prototyping and creating PCBs and other 2D designs. Furthermore, alternative DIY solutions may utilize less precise mechanical systems.

  - General Utility/application of the invention:

PCB prototyping

Creating stencils for electronics

Educational demonstrations of CNC technology

Generating 'handwritten-like' files and 2D diagrams

Note-taking automation

  - Advantages of the invention disclosing about the increased efficiency/efficacy :

1. Cost-effectiveness: 3D printing with the Ender 3 significantly reduces the cost of manufacturing components compared to traditional methods. This makes the technology more accessible.

2. Accessibility: Lower cost makes CNC/PCB plotting technology more accessible to a wider range of users, including hobbyists, small businesses, and educational institutions.

3. Ease of assembly: 3D printed parts can be designed for simpler assembly, reducing the time and skill required for construction.

4. Reduced Waste: Additive manufacturing can minimize material waste compared to subtractive manufacturing processes

5. Improved Precision: The use of finely polished, 3D-fabricated gears offers the potential for increased precision and reduced friction compared to pulley/rope-gear systems.

  - Best way of using the invention as well as possible variants :

    - Interchangeable tool heads for different applications (e.g., pens, cutters, laser engravers).

    - Enclosed design for safety and dust control.

    - Wireless control.

    - Integration with other design software.

  - Working of invention along with Drawing, schematics and flow diagrams if required with complete explanations:

  Design Creation: The user begins by creating their desired image or PCB layout using Inkscape, a vector graphics editor. This design is then saved as an SVG (Scalable Vector Graphics) file.

  G-code Generation: To prepare the image for the plotter, the SVG file is opened and further processed in JS Cut. This software translates the vector graphics into a set of G-code instructions, which dictate the precise movements required for plotting.

  Control: This generated G-code is then uploaded to the Arduino Uno microcontroller using UGS (Universal G-code Sender). The Arduino processes these instructions, meticulously controlling the movement of the plotter's X, Y, and Z axes.

  Mechanical System: Key mechanical components for this CNC/PCB plotter were fabricated using an Ender 3 3D printer. The plotter utilizes 3-28BJ stepper motors and drivers (one for each axis) to achieve precise movement.

  Output: By precisely coordinating the motion of these axes, the plotter's tool is guided along the exact path specified in the G-code, ultimately resulting in the creation of the intended drawing or PCB traces.

Flowchart:

- Have you conducted Primary Patent Search? Yes / No (if yes, attach the patent search results)

- List out the known ways about how others have tried to solve the same or similar problems? Indicate the disadvantages of these approaches. In addition, please identify any prior art documentation or other material that explains or provides examples of such prior art efforts

| S. No. | Existing state of art | Drawbacks in existing state of art | Overcome (how your invention is overcoming the drawback) |
| --- | --- | --- | --- |
| 1. | Commercially available CNC plotters/drawing robots | High cost, limiting accessibility | Our 3D-printed design significantly reduces manufacturing costs, making the technology more accessible.. |
| 2. | DIY CNC plotters using pulley/rope-gear systems | Potential for increased friction, wear and tear, and lower precision | Our design utilizes 3D-fabricated and finely polished gears, potentially leading to smoother operation and higher precision. |

- List the Technical features and Elements of the invention along with the Description of your invention from start to end.:

1. Technical Features and Elements

3-axis control (X, Y, Z) for precise movement.

3D-printed structural components for cost-effectiveness and customization.

G-code control for automated operation.

Arduino Uno microcontroller for processing instructions and controlling motors.

Stepper motors: 3-28BJ (for accurate motion).

Drivers: 3 (one for each axis) (to control the stepper motors).

Software:

- Inkscape (for creating designs in SVG format).

- JS Cut (for converting SVG files to G-code).

- UGS (Universal G-code Sender) (for sending G-code to the Arduino).

Description:

1. Design Phase: The process begins with the user creating a design using Inkscape. The design is saved as an SVG file.

2. G-code Conversion: The SVG file is then imported into JS Cut, where it's converted into G-code. G-code is a language that the Arduino can understand to control the plotter's movements.

3. G-code Transmission: UGS is used to send the generated G-code to the Arduino Uno.

4.  Motion Control: The Arduino Uno receives the G-code and sends signals to the stepper motor drivers. These drivers control the stepper motors, which move the plotter's tool head along the X, Y, and Z axes.

5. Plotting: The coordinated movement of the tool head, guided by the G-code instructions, results in the desired drawing or PCB traces on the material.

- List out the features of your invention which are believed to be new and distinguish them over the closest technology.

Cost-effective design through the use of 3D-printed components.

Integration of open-source software (Inkscape, JS Cut, UGS) for accessibility and customization.

Use of finely polished 3D-fabricated gears for potentially improved precision compared to pulley/rope       systems.

- Has the invention been built or tested or implemented? If yes please provide the Efficiency/Efficacy  details of the invention.

Yes, the invention successfully demonstrates the construction and operation of the 3-axis CNC/PCB Plotter

Efficiency/efficacy data:

The plotter successfully performs pen-based drawing and PCB tracing.

The plotter's interpolation property performs well, indicating smooth and accurate curved movements.

The design demonstrates a cost-effective alternative to commercial plotters by utilizing 3D-printed components.

10.   Briefly state when and how you first conceived this idea?

The idea was conceived with the goal of developing a more efficient and cost-effective method, with the ease of also making a make-shift writing/drawing machine, for creating drawings, diagrams, and PCB traces compared to existing commercial solutions.

11 Have you sold, offered for sale, publicly used or published anything related to this invention? If yes, please briefly explain the dates and circumstances. List those individuals to whom you have revealed your invention. Were non-discloser documents signed prior to discloser in each case? Please state any deadlines of which you may be aware for filing an application on this invention.

Yes, the invention was displayed during the Department’s Project Fair.

Students and Faculty of the ICT Department were present.

No, non-disclosure documents were not signed.

- Include any reasons that your invention would not have been obvious to someone of average skill in the art.

The invention's non-obvious aspects include:

- Innovative use of 3D printing for structural components to achieve cost reduction.

- Unique software workflow (Inkscape - JS Cut - UGS) for controlling the hardware.

- Potential precision improvement with 3D-fabricated gears over pulley/rope systems.

- Overall focus on cost-effectiveness and accessibility, distinguishing it from expensive commercial solutions.

- Additional comments by inventor (if you want to give more details out of scope of this IDF).

- Drawings/Flowchart/Table